Timing Libraries, Synthesis Approaches, and Efficient Flip-Flop Coding

The main aim of this lab is:

Understanding the .lib timing library used in open-source PDKs.

Comparing hierarchical and flat synthesis methods.

Timing Libraries:

SKY130 PDK Overview

The SKY130 PDK is an open-source Process Design Kit based on SkyWater Technology's 130nm CMOS technology. It provides essential models and libraries for integrated circuit (IC) design, including timing, power, and process variation information.

Decoding tt_025C_1v80 in the SKY130 PDK

tt: Typical process corner.

025C: Represents a temperature of 25°C, relevant for temperature-dependent performance.

1v80: Indicates a core voltage of 1.8V.

This naming convention clarifies which process, voltage, and temperature conditions the library models.

Hierarchical vs Flattened Synthesis:

Hierarchical Synthesis

It retains the module hierarchy as defined in RTL, synthesizing modules separately. Tools like Yosys process each module independently, using commands such as hierarchy to analyze and set up the design structure.

Advantages:

- Faster synthesis time for large designs.

- Improved debugging and analysis due to maintained module boundaries.

- Modular approach, aiding integration with other tools.

Disadvantages:

- Cross-module optimizations are limited.

- Reporting can require additional configuration.

Flattened Synthesis

It merges all modules into a single flat netlist, eliminating hierarchy. The flatten command in Yosys collapses the hierarchy, allowing whole-design optimizations.

Advantages:

- Enables aggressive, cross-module optimizations.

- Results in a unified netlist, sometimes simplifying downstream processes.

Disadvantages:

- Longer runtime for large designs.

- Loss of hierarchy complicates debugging and reporting.

- Can increase memory usage and netlist complexity

Asynchronous Reset D Flip-Flop:

module dff_async_res (input clk, input async_reset, input d, output reg q);

always @ (posedge clk, posedge async_reset)

if (async_reset)

q <= 1'b0;

else

q <= d;

endmodule

Asynchronous reset: Overrides clock, setting q to 0 immediately.

Edge-triggered: Captures d on rising clock edge if reset is low.

Asynchronous Set D Flip-Flop:

module dff_async_set (input clk, input async_set, input d, output reg q);

always @ (posedge clk, posedge async_set)

if (async_set)

q <= 1'b1;

else

q <= d;

endmodule

Asynchronous set: Overrides clock, setting q to 1 immediately.

Synchronous Reset D Flip-Flop

module dff_sync_res (input clk, input async_reset, input sync_reset, input d, output reg q);

always @ (posedge clk)

if (sync_reset)

q <= 1'b0;

else

q <= d;

endmodule

Synchronous reset: Takes effect only on the clock edge

Icarus Verilog Simulation

Compile:

iverilog dff_async_res.v tb_dff_async_res.v

Run:

./a.out

View Waveform:

gtkwave tb_dff_async_res.vcd