### Gate-Level Simulation (GLS), Blocking vs. Non-Blocking, and Synthesis-Simulation Mismatch

### 1. Gate-Level Simulation (GLS)

Gate-Level Simulation (GLS) is a vital verification step in the VLSI design flow. It involves simulating the synthesized gate-level netlist of a digital circuit to validate its functional correctness, timing behavior, power estimates, and test structures.

#### Why Perform GLS?

- Synthesis Validation: Ensures that the synthesis tools correctly translated the RTL (Register-Transfer Level) code into a gate-level netlist.

- Timing Verification: GLS can incorporate realistic timing delays from SDF (Standard Delay Format) files, allowing engineers to check for potential timing violations like setup and hold errors.

- Testability: It confirms that design-for-test (DFT) structures, such as scan chains, are functioning correctly after synthesis.

#### When is GLS Performed?

GLS is typically performed after the RTL code has been synthesized into a gate-level netlist but before the physical design (layout) phase. This allows for early detection of issues before they become more expensive to fix.

#### Types of GLS

- Functional GLS: A simulation that focuses solely on logical correctness, often ignoring or using zero/unit delays.

- Timing GLS: A more realistic simulation that includes annotated timing data to verify the circuit's real-world timing behavior.

2. Synthesis-Simulation Mismatch

A synthesis-simulation mismatch occurs when the behavioral simulation of the RTL code does not match the results of the gate-level simulation or the final hardware. This is a critical issue that can lead to silicon failures.

#### Common Causes

- Non-synthesizable Constructs: Using Verilog constructs that are not supported by synthesis tools, such as # delays, initial blocks, and force/release statements.

- Incomplete or Ambiguous Code: This includes missing else clauses in if-else statements, which can infer latches, or improper sensitivity lists in always blocks.

- Tool Interpretation Differences: Different simulators and synthesis tools may interpret ambiguous RTL code differently, leading to inconsistent results.

3. Blocking vs. Non-Blocking Assignments

Verilog provides two types of procedural assignments for use inside always and initial blocks: blocking (=) and non-blocking (<=).

#### Blocking Statements (=)

Blocking assignments are so named because they "block" the execution of subsequent procedural statements in the same always block until the assignment is complete.

- Execution: They execute sequentially and immediately.

- Use Case: Ideal for modeling combinational logic.

#### Non-Blocking Statements (<=)

Non-blocking assignments are scheduled to happen at the end of the current time step. The procedural statements following a non-blocking assignment can continue to execute without waiting for it to complete.

- Execution: They execute concurrently at the end of the current simulation time step.

- Use Case: Essential for modeling sequential logic (e.g., flip-flops). This prevents race conditions and ensures predictable behavior.

### 4. Lab Examples

#### Lab 1: Ternary Operator MUX

Verilog Code:

Verilog

module ternary_operator_mux (input i0, input i1, input sel, output y);

assign y = sel ? i1 : i0;

endmodule

Function: This code implements a simple 2:1 multiplexer. The output y is set to i1 if sel is 1, and to i0 if sel is 0.

#### Lab 2: Synthesis of MUX

#### Lab 3: Gate-Level Simulation of MUX

This lab focuses on performing a GLS of the synthesized MUX to verify its functionality. The simulation would use the generated netlist and a testbench.

#### Lab 4: Bad MUX Example (Common Pitfalls)

Verilog Code with Issues:

Verilog

module bad_mux (input i0, input i1, input sel, output reg y);

always @ (sel) begin

if (sel)

y <= i1;

else

y <= i0;

end

endmodule

Issues: This code has two major issues:

- Incomplete Sensitivity List: The always block should be sensitive to i0 and i1 as well. The correct list is always @ (*).

- Incorrect Assignment Type: A combinational circuit should use blocking assignments (=) to ensure immediate updates. Using non-blocking (<=) can lead to simulation mismatches.

Corrected Version:

Verilog

always @ (*) begin

if (sel)

y = i1;

else

y = i0;

end

#### Lab 5: GLS of Bad MUX

This lab demonstrates that simulating the bad_mux module will likely produce simulation warnings or mismatches compared to the synthesized gate-level netlist.

#### Lab 6: Blocking Assignment Caveat

Verilog Code:

Verilog

module blocking_caveat (input a, input b, input c, output reg d);

reg x;

always @ (*) begin

d = x & c;

x = a | b;

end

endmodule

Problem: The order of assignments is problematic. The statement d = x & c; will use the old value of x before x is updated by the statement x = a | b;. This can lead to a behavioral difference between simulation and synthesis.

Corrected Order:

Verilog

always @ (*) begin

x = a | b;

d = x & c;

end

#### Lab 7: Synthesis of the Corrected Module

This lab involves synthesizing the corrected version of the blocking_caveat module to see the expected and correct hardware implementation.