Introduction to Verilog RTL Design and Synthesis

I have gained hands-on knowledge of simulating Verilog codes using iVerilog and also worked on logic synthesis using Yosys.

Introduction:

Simulator: A simulator is a software tool that checks your digital circuit’s functionality by applying test inputs and viewing outputs. This helps you verify your design before hardware implementation.

Design: The design is your Verilog code describing the intended logic functionality

Testbench: A testbench is a simulation environment that applies various inputs to your design and checks if the outputs are correct

Simulating 2x1 Multiplexer:

Step 1: Install Required Tools

sudo apt install iverilog

sudo apt install gtkwave

Step 2: Simulate the Design

Compile the design and testbench: iverilog simple_mux.v tb_simple_mux.v

Run the simulation: ./a.out

View the waveform: gtkwave tb_simple_mux.vcd

Verilog Code Analysis:

module simple_mux (input i0, input i1, input sel, output reg y);

always @ (*)

begin

if(sel)

y <= i1;

else

y <= i0;

end

endmodule

Introduction to Yosys:

Yosys is a powerful open-source synthesis tool for digital hardware. It takes your Verilog code and converts it into a gate-level netlist.

Synthesis with Yosys:

- yosys

- read_liberty –lib /address/to/your/sky130/file/sky130_fd_sc_hd__tt_025C_1v80.lib

- read_verilog /home/address/to/verilog/file

- synth -top simple_mux

- abc -liberty /address/to/your/sky130/file/sky130_fd_sc_hd__tt_025C_1v80.lib

- show